CURRENT ELECTRICITY
JEE MAIN — CURATED QUESTION BANK

JNV Class XII Physics • Card 06 • Curated v1
Adapted from identified JEE Main PYQs

# How to use this bank

These are concise, student-facing adaptations of identified JEE Main Current Electricity PYQs. Numerical values, concepts and answers are retained; wording is normalized rather than reproduced verbatim. Each entry records its year/session/shift and the source used for verification.

1. 2026 • Apr 2 • Shift 2 • MCQ

Two resistors of 200 Ω and 400 Ω are in series across 100 V. A 200 V, 100 W bulb is connected across the 400 Ω resistor. Find the voltage across the bulb.

Answer / route: 50 V

2. 2026 • Apr 4 • Shift 1 • MCQ

A voltmeter has resistance x Ω and a 20 V maximum range. What series resistance is needed to extend its range to 30 V?

Answer / route: x/2 Ω in series

3. 2026 • Apr 5 • Shift 1 • MCQ

For the shown multi-loop circuit, determine the three branch currents I₁, I₂ and I₃ for the given 10 V and 5 V sources and resistor network.

Answer / route: I₁ = 2.5 A; I₂ = 1.875 A; I₃ = 1.875 A

4. 2026 • Apr 5 • Shift 1 • MCQ

A circuit contains a 9 Ω network made from the given resistor branches. Determine the current between terminals A and B for the specified 5 V-cell network.

Answer / route: 1.25 A

5. 2026 • Apr 5 • Shift 2 • Numerical

A cell gives 0.25 A through 5 Ω and 0.50 A through 2 Ω. Determine its internal resistance.

Answer / route: 1 Ω

6. 2026 • Apr 6 • Shift 1 • Numerical

In the given two-loop circuit, calculate the heat produced in the 6 Ω resistor during 100 s; the paper expresses the result as α/100 J.

Answer / route: α = 3477

7. 2026 • Apr 6 • Shift 2 • Numerical

A galvanometer gives full-scale deflection at 500 mA when shunted by 2 Ω. With a 470 Ω series resistor it gives full-scale deflection at 10 V. Find its coil resistance.

Answer / route: 50 Ω

8. 2026 • Apr 6 • Shift 2 • Numerical

Two cells (1 V, 2 Ω) and (2 V, 1 Ω) are connected in parallel and supply 1 A to an external resistor. Reversing one cell changes the current to α/5 A. Find α.

Answer / route: 3

9. 2026 • Apr 8 • Shift 2 • Numerical

For the specified steady-state resistor-capacitor network, determine the charge stored by the 100 μF capacitor.

Answer / route: 200 μC

10. 2025 • Apr 3 • Shift 2 • Numerical

Cells of 1 V, 2 Ω and 2 V, 1 Ω are first connected in series with 6 Ω. They are then connected in parallel with the same external resistance. Find the specified ratio of the two currents.

Answer / route: 4/3

11. 2025 • Apr 4 • Shift 2 • MCQ

n identical bulbs, each designed to consume power p independently, are connected in series across the same mains voltage. Find the total power consumed.

Answer / route: p/n

12. 2025 • Apr 4 • Shift 2 • MCQ

Using three 5 Ω resistors and one 10 Ω resistor in the arrangements shown, identify the arrangement that gives an equivalent resistance of 6 Ω.

Answer / route: The arrangement whose equivalent is 6 Ω (verify against the original figure)

13. 2025 • Jan 23 • Shift 2 • Numerical

An 8 μF capacitor is in parallel with a 10 Ω resistor; this combination is in series with a 15 Ω resistor and a 5 V source. Find the steady-state capacitor charge.

Answer / route: 16 μC

14. 2025 • Jan 24 • Shift 1 • Numerical

A 9 Ω wire is bent into an equilateral triangle. Find the equivalent resistance between any two vertices.

Answer / route: 2 Ω

15. 2025 • Jan 28 • Shift 1 • MCQ

A wire of resistance R is formed into an equilateral triangle and an identical wire into a square. Find the ratio of resistance measured between the endpoints of an edge in the two shapes.

Answer / route: 32/27

16. 2025 • Jan 28 • Shift 1 • MCQ

Three resistors, each r/3, are connected in the configuration shown. Determine the equivalent resistance between the two terminals.

Answer / route: r/9

17. 2025 • Jan 28 • Shift 2 • Numerical

In the given bridge network, potentials at points A and B are equal. Determine the current I for the specified resistor values.

Answer / route: Determine using bridge-balance condition; original figure required

18. 2024 • Current Electricity • Numerical

Twelve 2 Ω wires form a cube. A 6 V battery is connected across two specified vertices. Determine the resulting circuit current.

Answer / route: Requires the original cube connection/vertices

19. 2024 • Current Electricity • MCQ

A 50 W, 200 V bulb is operated from a 100 V supply. Assuming resistance remains constant, find its power dissipation.

Answer / route: 12.5 W

20. 2024 • Current Electricity • Numerical

Two wires of the same material have equal mass but different dimensions. Determine the requested resistance relation from their lengths and cross-sectional areas.

Answer / route: Use R = ρL/A with equal mass constraint

21. 2024 • Current Electricity • MCQ

A potentiometer is used to determine a battery's internal resistance. For a known external resistance, the balance length is measured. Determine the internal resistance using the given balance-length data.

Answer / route: Use r = R(l₁−l₂)/l₂

22. 2021 • Current Electricity • MCQ

Two measured resistances are (4 ± 0.8) Ω and (4 ± 0.4) Ω. Determine the uncertainty in their parallel combination using the required error-propagation method.

Answer / route: Requires the exact requested uncertainty form from the original paper

23. 2020 • Current Electricity • MCQ

Among copper, tungsten, mercury and aluminium, identify the material with the required resistivity relationship specified in the original question.

Answer / route: Use standard resistivity ordering; verify exact options in original paper

24. 2006 • Current Electricity • MCQ

A material B has twice the specific resistance of material A. A wire of B has twice the diameter of an A wire. Compare their resistances for the stated length condition.

Answer / route: Use R = ρL/A

25. 2003 • Current Electricity • MCQ

A potentiometer wire is 100 cm long and a standard cell of emf E is used. Determine the requested unknown emf from the stated balance-length information.

Answer / route: Use V ∝ balance length

# Source and verification notes

Primary provenance: official JEE Main/NTA examination documents and final answer-key material. Chapter-level discovery and cross-checking: JEEPrep and PrepSharp. Figure-dependent questions are explicitly marked where the original figure is essential; they should not be presented as figure-free questions in the final student bank.

NTA: https://jeemain.nta.nic.in/documents/

JEEPrep Current Electricity chapter index: https://jeeprep.app/chapters/current-electricity

PrepSharp 2026 Current Electricity index: https://www.prepsharp.in/questions/mains/physics/current-electricity/year/2026